Put the tool on the desktop.

| Input Parameters | Parameter Description | Output | Output status |
| --- | --- | --- | --- |
| IP Address | IP address of NVR eg：’192.168.1.30’ | Current Time | The number of times that packets were sent |
| NVR Port | The receive port of NVR consistent with NVR configuration，not recommend 0 | Status | The status of sending packets：Sending、Stopping、error[key] |
| Socket Type | The type of sending packets TCP and UDP are available, recommend UDP | Date | The date of sending packets |
| Source Port | The PC source port of TCP consistent with NVR configuration,recommend 0 | Start | Start Button |
| Time Interval | The interval of sending packets | Stop | Stop Button |
| Text | Packets sending data |  |  |

Imperfection：

1、You have to wait about 20 seconds when you are trying to stop the thread because of the delay.

2、The limit of TCP. Local ports is not supposed to be bound repeatedly, so we recommend inputting 0 or modifying the Source Port every time you send the TCP packets.

3、The TIME_WAIT of TCP cause the problem that socket would disappear after a period of time. Sending the TCP packets repeatedly may cause the errors like 10067.

Status：10048            ——Modify source Port

Status：10061            ——Check the port of NVR configuration and the tool

The solution when running errors happen.

The running error as shown in the pic below.

Solution：

Operate as follows on your pc:

Start——Operation——regedit  Delet the key values in the position shown below. HKEY_CURRENT_USER\Software\Microsoft\Windows\CurrentVersion\Run HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\Windows\CurrentVersion\Run

Reference solution Sites：https://zhidao.baidu.com/question/266538447.html

You can solve the other problems by rebooting the PC.